# Wood parts for 2 ORION-3.5 speaker cabinets

## 1 - Top baffle for midrange and tweeters

| Part | Qty | Size (inch) | Material | Use |
| --- | --- | --- | --- | --- |
| A | 2 | 11.5 x 21.5 x 1/2 | Plywood | Midrange & tweeter baffle with 2 holes |
| B | 2 | 10.5 x 18.25 x 1/4 | Birch Plywood | Dress panel with 2 holes |
| C | 2 | 11.5 x 3.5 x 3/4 | Plywood | Brace for mounting to woofer |
| D | 4 | 20 x 3/4 x 3/4 | Hardwood | Corner brace for side panel mounting |
| Q | 4 | 6 x 1 x 3/4 | Plywood | Spacer for rear tweeter baffle |
| R | 2 | 11.5 x 6 x 3/4 | Plywood | Rear tweeter baffle |

24 x 24 x 1/2 sheet of multi-ply

24 x 24 x 1/4 sheet of birch plywood
84 x 3/4 x 3/4 oak

## 2 - W-frame for woofer

| Part | Qty | Size | Material | Use |
| --- | --- | --- | --- | --- |
| E | 4 | 24.5 x 13.5 x 1/4 | Hardboard, Masonite | Side cover |
| F | 8 | 11 x 13.5 x 3/4 | Plywood | Shelf |
| G | 2 | 11 x 6.75 x 3/4 | Plywood | Brace |
| H | 2 | 11 x 9 x 3/4 | Plywood | Brace |
| J | 2 | 11 x 5.75 x 3/4 | Plywood | Brace |

48 x 48 x 3/4 sheet

## 3 - Side panels

| Part | Qty | Size | Material | Use |
| --- | --- | --- | --- | --- |
| K | 2 + 2 | 46-3/8 x 16 x 3/4 | Birch plywood (8 ply) | Wood finished side panels, L & R |
|  |  |  | Plywood or MDF | Painted side panels, L & R |
|  | 1 | 46-3/8 x 16 x 1/8 | Hardboard, Masonite | Template |

48 x 48 x 3/4 sheet
48 x 24 x 1/8 sheet

## 4 - Dress and grill covers

| Part | Qty | Size | Material | Use |
| --- | --- | --- | --- | --- |
| L | 2 | 10.5 x 20.0 x 1/8 | Hardboard | Dress cover |
| M | 2 | 10.5 x 1.0 x 1/2 | Plywood | Cover support |
| N | 2 | 10.5 x 1.0 x 1/8 | Hardboard, Masonite | Cover retainer |

48 x 48 x 1/8 sheet

Total: 15 different parts, 44 pieces